Note: The original file is: BorgWarner_GEN3.0 EEPROM layout – UHE_isoSPI_Anforderungsdokument.xlsx

SHEET 0: Cover Page

Project: UHE isoSPI Date: 43964

Change History

| Version | Change Description | Status | Author | Date |
| --- | --- | --- | --- | --- |
| 1.0.0 | Initial | In Progress | RRa | 43780 |
| 1.0.1 | EEPROM requirements added |  |  |  |
| 1.0.2 | Requirements for safety measures added (with RRa, DWe, MFl, JEr) | In Progress | WIn | 43964 |
| 1.0.3–1.0.5 | Requirements for safety measures (with RRa, DWe, MFl, JEr) | In Progress | WIn | ... |
| 1.0.6 | EEPROM memory layout revised | In Progress | PSc | 43991 |
| 1.0.7–1.0.13 | Various updates to safety requirements and cell balancing | In Progress | ... | ... |
| 1.0.14 | "Department Requirements" updates (Requirement IDs added, references to Safety Manual, several IDs removed) | In Progress | RMr | 44048 |
| 1.0.15 | Update of requirements | In Progress | Rra, Nra, PSc | 44694 |

Document Release

| Version | Review Proof | Review | Release | Date |
| --- | --- | --- | --- | --- |

Document Description Document Number Document Name

Open Points

Release Process

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Created by |  |  |  |
| Reviewed by |  |  |  |
| Released by |  |  |  |

SHEET 1: Customer - System Requirements

Legend:

- 0 = Must

- 1 = Can

- Inf = Information

QSPI Interface

- AK.UHE.SW.0001 (0): The isoSPI interface shall be addressed via QSPI3. The CS (chip select) for the standard isoSPI shall be controlled via P23.5 and the reverse isoSPI via P23.4.

- Info01: The QSPI module baud rate is fixed at 1 Mb/s.

- Info02: The QSPI module is operated with CPHA = 1 and CPOL = 0.

LTC681x Configuration

- AK.UHE.SW.0002 (1): The LTC681x ADCs shall be operated in normal mode at 7 kHz.

- Info03: No cell is discharged at startup.

- Info04: Discharge Timeout is disabled at startup.

- AK.UHE.SW.0003 (0): All internal GPIO pull-downs of the LTC681x shall be configured as inactive at all times.

- AK.UHE.SW.0004 (1): Maximum cell voltage = 4.3 V, minimum cell voltage = 2.5 V. These values shall be written to the LTC681x configuration registers.

- AK.UHE.SW.0005 (0): The LTC681x shall remain permanently in REFUP state.

isoSPI Module

- AK.UHE.SW.0006 (1): Task period shall be 1 second.

- AK.UHE.SW.0007 (0): The isoSPI task shall run on Core 1.

- AK.UHE.SW.0008 (0): Number of modules, cells to be measured, and LTC ICs per module shall be configurable via macros.

- AK.UHE.SW.0009 (0): The isoSPI task shall send CAN messages according to the adapted "AkaCAN_Matrix_BMU_UHE.xlsx".

- AK.UHE.SW.0010 (0): The isoSPI task shall receive CAN messages for writing data to the module EEPROM.

- AK.UHE.SW.0011 (0): The isoSPI task shall send internal messages containing current module data and error flags to BMS-M and SCU.

- AK.UHE.SW.0054 (0): All values shall be stored as u16 except: Temperatures (float), Currents (s32), over-/undervoltage flags (u32), thermal shutdown occurred (bool).

- AK.UHE.SW.0012 (0): Temperature sensors shall be mapped to cell voltages according to the defined assignment.

- AK.UHE.SW.0013 (0): Shunt current sensors shall support offset calibration... (requirement partially withdrawn).

Error Handling & Communication

- AK.UHE.SW.0014 (0): If no poll result is received within 5 ms → set BMU_ISOSPI_Error_Timeout_Saf.

- AK.UHE.SW.0035 (0): All data sent to/received from LTC681x shall be protected with CRC. On CRC error, set error flag and use value 0xFFAF.

EEPROM

- AK.UHE.SW.0038 (0): The EEPROM memory layout on the modules shall be implemented as specified.

- AK.UHE.SW.0053 (0): Safety Area uses CRC16 (Poly 0xAC9A). Other areas use CRC32.

- AK.UHE.SW.0039 (0): A mirrored copy of all EEPROM contents shall be maintained in the BMU (mirrored EEPROM).

- AK.UHE.SW.0040 (0): At BMU startup, all EEPROM contents (standard pages + ID Page) shall be copied to the mirrored EEPROM.

- AK.UHE.SW.0044 (0): Via CAN message Mxxx_StartLTCTransmission, the following actions can be triggered (ActionIndex):

  - 0: Write Safety Area

  - 1: Write Static Area

  - 2: Write Dynamic Area

  - 3: Re-read all EEPROMs

  - 4: Read ID Page status

  - 5: Lock all ID Pages

  - 6: (EOL only) Initialize Dynamic Area

Cell Balancing

- AK.UHE.SW.0058 (0): Cell balancing shall use the S-Pins (S-Pin/PWM register groups).

- AK.UHE.SW.0061 (0): Balancing shall be disabled during cell voltage measurement using MUTE/UNMUTE commands.

SHEET 2: Department Requirements (Internal)

Safety Mechanisms

- REQ_LTC_SM_001 (0): MUX Decoder Check shall be performed at least every x seconds.

- REQ_LTC_SM_004 (0): VREF2 voltage shall be measured cyclically (2.990 V – 3.014 V).

- REQ_LTC_SM_006 (0): VREG voltage shall be measured cyclically (4.5 V – 5.5 V).

- REQ_LTC_SM_007 (0): VREGD voltage shall be measured cyclically (2.7 V – 3.6 V).

- REQ_LTC_SM_009 (0): Open-wire check CV shall be performed cyclically.

- REQ_LTC_SM_017 (0): Overlap cell measurement shall be performed with the following maximum deviations (V1.6):

  - < 0.8 V: 1.2 mV

  - 0.8–2 V: 2.3 mV

  - 2–3.3 V: 3.4 mV

  - 3.3–4.2 V: 4.2 mV

    - 5 V: 7.0 mV

- REQ_LTC_SM_019 (0): CRC of every received isoSPI data packet shall be verified.

- REQ_LTC_SM_022 (0): All cell voltages shall be checked against allowed range.

- REQ_LTC_SM_024 (0): All cell temperatures shall be checked against allowed range.

- REQ_LTC_SM_027 (0): Sum of cell voltages shall be plausibilized against module voltage.

- REQ_LTC_SM_029 (0): Discharge circuit function check (voltage drop verification).

- REQ_LTC_SM_030 (0): Short circuit check on GPIOs connected to temperature sensors.

- REQ_LTC_SM_032 / REQ_LTC_SM_033 etc.: Various open-wire and pull-up/pull-down checks.

General

- No retries shall be performed during safety mechanism execution in case of errors.

- Dedicated error flags shall be defined for safety mechanism failures.

SHEET 3: Measures

- EN 50272-3, 10.1 → Adapt BMS cable routing

- ECE R100 → Test setup for determining ISO resistance

SHEET 4: Description

ID Assignment Rules

External Requirement: AAA.BBBB.EX.CC.xxxx

Internal Requirement: AAA.BBBB.CC.xxxx

Change Marking

- White: No change

- Light Blue: Formal change only

- Orange: New or changed requirement

- Gray + Strikethrough: Deleted

- Yellow: Not yet valid / to be detailed

SHEET 7: EEPROM Content Flags

Initial Safety Checks

| Area | A+B Bad CRC | A Bad CRC | B Bad CRC | A ≠ B (correct CRC) | ID Page not locked |
| --- | --- | --- | --- | --- | --- |
| Dynamic | CompleteAreaCRCMismatchDyn | SingleAreaCorrupted | SingleAreaCorrupted | - | - |
| Static | CompleteAreaCRCMismatchSafStat | SingleAreaCorrupted | SingleAreaCorrupted | AreaMismatch | - |
| Safety | CompleteAreaCRCMismatchSafStat | SingleAreaCorrupted | SingleAreaCorrupted | AreaMismatch | IDPageNotLocked |

SHEET 9: EEPROM Memory Layout (1kByte M24C08)

ID Page (Safety Area A) – 16 bytes

- CRC16

- Cell Type (Major / Minor)

- Module Type (Major / Minor)

- Module Serial Number (AKASOL)

- PCB Type (Major / Minor)

Static Area 1 & 2 (160 bytes each)

- Topology (series cells, parallel cells)

- PCB Serial Number

- IC Type

- 2nd Life ID

- Locking recognition

- Reserved

Dynamic Area 1 & 2 (336 bytes each)

- CRC32 + Write Counter

- Unique ID (Battery Pack SN)

- Time integral of battery alive [h]

- Lifetime Discharge Ah counter

- Overvoltage / Undervoltage / Overtemperature / Overcurrent counters (multiple thresholds)

- Project-specific data (derating factors, SoH_modulelevel, etc.)

- Reserved

EEPROM Type: M24C08-A125 (1 kB + 16-byte ID Page)

| Memory Area | Size [bytes] | Offset (dec) | Offset (hex) | Total Area Size | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ID Page (Safety Area A) | 16 | 0 | 0x0000 | 16 | CRC16 + Configuration Data (Safety relevant) | Can be permanently locked |
| Cell Type Major | 1 | 2 | 0x0002 | - | Cell type (major) | e.g. 36 |
| Cell Type Minor | 1 | 3 | 0x0003 | - | Cell type (minor/revision) | - |
| Module Type Major | 1 | 4 | 0x0004 | - | Module type (major) | - |
| Module Type Minor | 1 | 5 | 0x0005 | - | Module type (minor/revision) | - |
| Module Serial Number | 8 | 6 | 0x0006 | - | AKASOL Module Serial Number | - |
| PCB Type Major | 1 | 14 | 0x000E | - | PCB type (major) | - |
| PCB Type Minor | 1 | 15 | 0x000F | - | PCB type (minor) | - |

Static Area 1 & Static Area 2 (redundant)

| Field | Size [bytes] | Offset (Static 1) | Description |
| --- | --- | --- | --- |
| CRC32 | 4 | 0x0010 / 0x00B0 | CRC32 of the area |
| Number of serial cells | 1 | 0x0014 / 0x00B4 | Topology: cells in series |
| Number of parallel cells | 1 | 0x0015 / 0x00B5 | Topology: cells in parallel |
| PCB Serial Number | 8 | 0x0016 / 0x00B6 | PCB serial number |
| IC Type Major / Minor | 2 | 0x001E / 0x00BE | LTC IC type |
| 2nd Life ID | 8 | 0x0020 / 0x00C0 | 2nd Life identification |
| Locking Recognition | 1 | 0x0028 / 0x00C8 | Locking status |
| Reserved | 135 | 0x0029 / 0x00C9 | For future use |

Dynamic Area 1 & Dynamic Area 2 (alternating write)

| Field | Size [bytes] | Offset (Dyn 1) | Description |
| --- | --- | --- | --- |
| CRC32 | 4 | 0x0150 | CRC32 of the area |
| Write Counter | 4 | 0x0154 | Incremented on every write |
| Unique ID (Battery Pack SN) | 8 | 0x0158 | Pack serial number |
| Time integral battery alive | 4 | 0x0160 | [hours] |
| Lifetime Discharge Ah counter | 4 | 0x0164 | Total discharged Ah |
| Overvoltage Counter 1 (4.25V) | 1 | 0x0168 | Counter for OV level 1 |
| Overvoltage Counter 2 (4.3V) | 1 | 0x0169 | Counter for OV level 2 |
| Undervoltage Counter 1/2 | 2 | 0x016A | UV counters |
| Overtemperature Counter 1/2 | 2 | 0x016C | OT counters |
| Overcurrent (discharge/charge) | 8 | 0x016E | Multiple current threshold counters |
| Project specific data | 4+ | 0x0172 | Derating factors, SoH_modulelevel, etc. |
| Reserved | 294 | 0x017A | For future use |

SHEET 13: isoSPI Error Flags (Detailed)

| Flag Name | Bit | Trigger Condition | Severity / Consequence | Reset Condition |
| --- | --- | --- | --- | --- |
| Bad CRC (per IC) | 1-4 | CRC mismatch on data from IC1–IC4 | Data set to 0xFFEF | Start of next measurement |
| Timeout | 5 | ADC polling timeout (>5ms) | Keep old values | - |
| Complete Safety Area CRC Error | 6 | Both Safety Areas A+B have bad CRC | ERROR, stop EEPROM writes | Retry initial read |
| Complete Static Area CRC Error | 7 | Both Static Areas A+B have bad CRC | ERROR, stop EEPROM writes | Retry initial read |
| Complete Dynamic Area CRC Error | 8 | Both Dynamic Areas A+B have bad CRC | ERROR (limp-home possible) | Retry initial read |
| Single Safety Area CRC Error | 10 | One Safety Area corrupted | WARNING | Retry initial read |
| Single Static Area CRC Error | 11 | One Static Area corrupted | WARNING | Retry initial read |
| Single Dynamic Area CRC Error | 12 | One Dynamic Area corrupted | WARNING | After write / retry read |
| Safety Area Mismatch | 13 | A and B have correct CRC but content differs | WARNING | Retry initial read |
| Static Area Mismatch | 14 | A and B have correct CRC but content differs | WARNING | Retry initial read |
| ID Page Not Locked | 15 | Safety Area (ID Page) is not locked | WARNING | When successfully locked |
| I2C Communication Fail | 16 | Error in I²C communication with EEPROM | Data not read/written | Begin write / retry read |
| EEPROM Read Fail | 17 | Error reading full EEPROM during init | ERROR, write protection active | Retry initial read |
| Saf/Stat/Dyn Area Write Fail | 19-21 | Error while writing area | ERROR | Begin next write |
| Verify Write Fail | 25-27 | Written data != read-back data | ERROR | Begin next write |

SHEET 10: BMU Memory Allocation

Detailed flash memory map for the BMU (sectors S0–S26, sizes, offsets, write permissions, bootloader areas, application areas, configuration data, etc.).

SHEET 13: isoSPI Error Flags

Main Error Flags:

- BMU_ISOSPI_Error_Bad_CRC

- BMU_ISOSPI_Error_Timeout

- BMU_ISOSPI_Error_Init_Fail

- BMU_ISOSPI_Error_Cell_Regs_Not_Cleared

- BMU_ISOSPI_I2C_Communication_Fail

- IDPageNotLocked

- SafAreaWriteFail / StatAreaWriteFail / DynAreaWriteFail

- VerifySafWriteFail / VerifyStatWriteFail / VerifyDynWriteFail

- complSafAreaCRCErr, singleSafAreaCRCErr, safAreaMismatch, etc.

Each flag includes trigger condition, consequence, set/reset behavior.

Comprehensive Signal & Message List Extracted and compiled from the entire document (all sheets).

1. CAN Message Overview (AkaCAN)

| Message Name | Direction | Description / Content | Module-wise? | Notes |
| --- | --- | --- | --- | --- |
| Mxxx_BMSInfo | TX | Error flags, status per module | Yes | EEPROM + isoSPI errors |
| Mxxx_ModuleInformation | TX | CellType, Topology per module | Yes | - |
| Mxxx_SlaveSerials | TX | Module serial number (high + low) | Yes | - |
| Mxxx_WriteEEPROMData | RX | Write data to Dynamic Area | Yes | MUXed |
| Mxxx_ConfigurationMessage | RX | Write Safety Area / Static Area | Yes | - |
| Mxxx_StartLTCTransmission | RX | Trigger EEPROM actions (ActionIndex 0–6) | All modules | Key command |
| Mxxx_EEPROMChangeDynArea | RX | Select next Dynamic Area (A or B) | - | - |
| Mxxx_EEPROMData | TX/RX | Dynamic Area data transmission | Yes | MUXed |
| Start_Curr_Sens_Calib | RX | Trigger current sensor offset calibration | - | (Partially withdrawn) |

2. Measurement & Status Signals (TX)

| Signal Name | Unit | Description |
| --- | --- | --- |
| Mxxx_act_U_DigiPower_IC1_mV / IC2_mV | mV | VREF2, VREG, VREGD for IC1 & IC2 |
| Cell voltages (per cell) | mV | Individual cell voltages |
| Cell temperatures (mapped) | °C | Temperatures (float) |
| Currents (Shunt sensors) | A | s32, with offset calibration |
| Module voltage (sum / direct) | mV | For plausibility checks |
| Over/Undervoltage flags | u32 bitfield | Per cell |
| Thermal shutdown occurred | bool | - |

3. EEPROM-Related Signals & Flags

Dynamic Area Signals (transmitted via Mxxx_EEPROMData / BMSInfo):

- Write Counter

- Unique ID (Battery Pack SN)

- Time integral of battery alive [hours]

- Lifetime Discharge Ah counter

- Overvoltage counters (levels 1 & 2)

- Undervoltage counters (levels 1 & 2)

- Overtemperature counters (levels 1 & 2)

- Overcurrent counters (discharge/charge, multiple thresholds)

- Derating factors (deratefac_cp, deratefac_dcp)

- SoH_modulelevel

4. Complete isoSPI Error Flags List

| Flag Name | Type | Description |
| --- | --- | --- |
| BMU_ISOSPI_Error_Bad_CRC | Error | CRC mismatch on LTC data |
| BMU_ISOSPI_Error_Timeout | Error | Polling timeout (>5 ms) |
| BMU_ISOSPI_Error_Init_Fail | Error | Initialization failure |
| BMU_ISOSPI_I2C_Communication_Fail | Error | EEPROM I²C error |
| BMU_ISOSPI_Error_Too_many_Modules | Error | Too many modules detected |
| complSafAreaCRCErr | Error | Both Safety Areas corrupted |
| complStatAreaCRCErr | Error | Both Static Areas corrupted |
| complDynAreaCRCErr | Error | Both Dynamic Areas corrupted |
| singleSafAreaCRCErr | Warning | One Safety Area corrupted |
| singleStatAreaCRCErr | Warning | One Static Area corrupted |
| singleDynAreaCRCErr | Warning | One Dynamic Area corrupted |
| safAreaMismatch / statAreaMismatch | Warning | Redundant areas don't match (despite good CRC) |
| IDPageNotLocked | Warning | Safety Area (ID Page) not locked |
| SafAreaWriteFail / StatAreaWriteFail / DynAreaWriteFail | Error | Write failure |
| VerifySafWriteFail etc. | Error | Read-back verification failure |
| EEPROMReadFail | Error | Initial EEPROM read failure |
| IDPageStatusReadFail | Error | ID Page status read failure |

Safety Mechanism Error Flags (from Sheet 2):

- SM_badCRC

- SM_pollingTimeout

- SM_digRedFail

- SM_ClrRegsFail

- SM_CmdVerFail

5. ActionIndex for Mxxx_StartLTCTransmission

| Index | Action |
| --- | --- |
| 0 | Write Safety Area |
| 1 | Write Static Area |
| 2 | Write Dynamic Area |
| 3 | Re-read all EEPROMs + check |
| 4 | Read ID Page status |
| 5 | Lock all ID Pages |
| 6 | (EOL only) Initialize Dynamic Area |

SHEET 1: Customer - System Requirements

Legend:

- 0 = Must requirement

- 1 = Can / Optional

- Inf = Information

QSPI Interface

| Req. ID | Type | Description |
| --- | --- | --- |
| AK.UHE.SW.0001 | 0 | The isoSPI interface shall be addressed via QSPI3. CS for standard isoSPI via P23.5, reverse isoSPI via P23.4. |
| Info01 | Inf | QSPI baud rate is fixed at 1 Mb/s. |
| Info02 | Inf | QSPI operated with CPHA = 1 and CPOL = 0. |

LTC681x Configuration

| Req. ID | Type | Description |
| --- | --- | --- |
| AK.UHE.SW.0002 | 1 | LTC681x ADCs shall operate in normal mode at 7 kHz. |
| AK.UHE.SW.0003 | 0 | All internal GPIO pull-downs of LTC681x shall be permanently inactive. |
| AK.UHE.SW.0004 | 1 | Max cell voltage = 4.3 V, Min cell voltage = 2.5 V. These limits shall be written to the configuration registers. |
| AK.UHE.SW.0005 | 0 | LTC681x shall remain permanently in REFUP state. |

isoSPI Module

| Req. ID | Type | Description |
| --- | --- | --- |
| AK.UHE.SW.0006 | 1 | Task period shall be 1 second. |
| AK.UHE.SW.0007 | 0 | isoSPI task shall run on Core 1. |
| AK.UHE.SW.0008 | 0 | Number of modules, cells, and LTC ICs per module shall be configurable via macros. |
| AK.UHE.SW.0009 | 0 | Shall send CAN messages according to "AkaCAN_Matrix_BMU_UHE.xlsx". |
| AK.UHE.SW.0010 | 0 | Shall receive CAN messages to write data into the module EEPROM. |
| AK.UHE.SW.0011 | 0 | Shall send internal messages with module data and error flags to BMS-M and SCU. |
| AK.UHE.SW.0054 | 0 | Data types: most values u16, except Temperatures (float), Currents (s32), OV/UV flags (u32), thermal shutdown (bool). |
| AK.UHE.SW.0014 | 0 | If no poll result within 5 ms → set BMU_ISOSPI_Error_Timeout_Saf. |
| AK.UHE.SW.0035 | 0 | All data exchanged with LTC681x must be CRC-protected. On CRC error, set error flag and use value 0xFFAF. |

EEPROM Requirements (Key ones)

| Req. ID | Type | Description |
| --- | --- | --- |
| AK.UHE.SW.0038 | 0 | EEPROM memory layout on modules shall be implemented as specified (see Sheet 9). |
| AK.UHE.SW.0053 | 0 | Safety Area: CRC16 (Poly 0xAC9A). Other areas: CRC32. |
| AK.UHE.SW.0039 | 0 | BMU shall maintain a mirrored EEPROM copy of all module EEPROMs. |
| AK.UHE.SW.0040 | 0 | At startup, copy all EEPROM data (incl. ID Page) to mirrored EEPROM. Set error flags on read failure. |
| AK.UHE.SW.0044 | 0 | Via Mxxx_StartLTCTransmission (ActionIndex): 0=Write Safety, 1=Write Static, 2=Write Dynamic, 3=Re-read all, 5=Lock ID Pages, etc. |

Cell Balancing

| Req. ID | Type | Description |
| --- | --- | --- |
| AK.UHE.SW.0058 | 0 | Cell balancing shall use S-Pins (S-Pin/PWM registers). |
| AK.UHE.SW.0061 | 0 | Balancing shall be disabled during cell voltage measurement using MUTE / UNMUTE (with 1 ms delay). |

(Many more requirements exist in the original sheet — let me know if you want the complete 80+ row version.)

SHEET 2: Department Requirements (Internal / Safety)

Legend: 0 = Must

| Req. ID | Safety Manual ID | Description |
| --- | --- | --- |
| REQ_LTC_SM_001 | 5.3_SM1 | MUX Decoder Check shall be performed periodically (interval x to be defined). |
| REQ_LTC_SM_002 | 5.3_SM1 | On failed MUX Decoder Check, set error flag in BMS_Info. |
| REQ_LTC_SM_004 | 5.3_SM2 | Cyclically measure VREF2 (must be 2.990 V – 3.014 V). |
| REQ_LTC_SM_005 | 5.3_SM2 | Send VREF2 (IC1 & IC2) via CAN signals Mxxx_act_U_DigiPower_IC1_mV / _IC2_mV. |
| REQ_LTC_SM_006 | 5.3_SM3 | Cyclically measure VREG (4.5 V – 5.5 V). |
| REQ_LTC_SM_007 | 5.3_SM3 | Cyclically measure VREGD (2.7 V – 3.6 V). |
| REQ_LTC_SM_009 | 5.3_SM4 | Perform open-wire check on CV (cell voltages) periodically. |
| REQ_LTC_SM_014 | 5.3_SM7 | Digital filter & memory check CV (Fast Mode, values 0x9565 / 0x6A9A). |
| REQ_LTC_SM_015 | 5.3_SM11 | Digital filter & memory check AUX (Fast Mode). |
| REQ_LTC_SM_017 | 5.3_SM8 | Overlap cell measurement in Normal Mode (7 kHz) with defined max deviation (see V1.6 limits below). |
| REQ_LTC_SM_019 | 5.3_SM16 | Verify CRC of every received isoSPI packet. |
| REQ_LTC_SM_020 | 5.3_SM16 | All sent commands/data shall be CRC-protected. |
| REQ_LTC_SM_022 | 5.3_SM18 | Check all cell voltages against allowed range (Error code 1 = undervoltage, 2 = overvoltage). |
| REQ_LTC_SM_024 | 5.3_SM20 | Check all cell temperatures against allowed range (different error codes for charge/discharge). |
| REQ_LTC_SM_027 | 5.3_SM23 | Plausibilize sum of cell voltages vs. measured module voltage. |
| REQ_LTC_SM_029 | 5.3_SM27 | Discharge circuit functional check (measure voltage drop when discharge enabled). |
| REQ_LTC_SM_030 | 5.3_SM29 | Open-wire check on cell voltage channels. |
| REQ_LTC_SM_032 | 5.3_SM30 | GPIO short circuit check for temperature sensors (pull-up/pull-down test). |
| REQ_LTC_SM_045 | - | No retries during safety mechanism execution on communication errors. |
| REQ_LTC_SM_046 | - | Define dedicated error flags for safety mechanism failures (SM_badCRC, SM_pollingTimeout, SM_digRedFail, etc.). |

Overlap Cell Measurement Limits (REQ_LTC_SM_017 - V1.6)

- U_cell < 0.8 V → max diff 1.2 mV

- 0.8 – 2 V → 2.3 mV

- 2 – 3.3 V → 3.4 mV

- 3.3 – 4.2 V → 4.2 mV

  - 5 V → 7.0 mV