BMU Replacement – Interface Definition Template

Purpose: Use this template to define the information needed from the existing product team so firmware, hardware, and test development can evaluate and implement a replacement BMU.

Recommended scope: request more than a simple signal list. The package should include communication behavior, state transitions, error handling, configuration data, and EEPROM/NVM layout in addition to physical I/O.

1. High-Level Interface Diagram

| External tester / AkaTool | → CAN / diagnostics / test commands | Replacement BMU | → isoSPI / control / measurements / faults | CSC board(s) | → I2C / NVM access / parameter storage | EEPROM / NVM |
| --- | --- | --- | --- | --- | --- | --- |
| Fill in physical interface, CAN IDs, message names, timing, test sequences |  | Fill in BMU state machine, command handling, balancing logic, diagnostics |  | Fill in AFE count, cell map, GPIO/NTC map, balancing channels, protection logic |  | Fill in memory map, data ownership, update rules, CRC/checksum, defaults |

Tip: This diagram is only the front page view. The real handoff should be backed by the detailed tables below.

2. Information Request Checklist

| Area | What to Request | Why It Matters | Provided? |
| --- | --- | --- | --- |
| Tester ↔ BMU | Physical interface, CAN IDs, DBC/message list, byte layout, mux usage, expected command/response flow, timing and retry rules | Needed to implement the external control/diagnostic interface and test workflow | ☐ |
| BMU ↔ CSC | isoSPI topology, number of AFE devices, wake-up/startup sequence, polling sequence, fault reporting, balancing commands | Needed to bring up the CSC stack and match existing system behavior | ☐ |
| CSC hardware map | Cell-to-AFE mapping, temperature input map, balancing channel map, GPIO usage, unused pins, protection or interlock signals | Needed for firmware configuration and validation | ☐ |
| EEPROM / NVM | Memory map, parameter meanings, default values, scaling, write/update rules, integrity checks, manufacturing/programming flow | Needed to preserve calibration/configuration compatibility | ☐ |
| System behavior | State machine, startup/shutdown conditions, fault reactions, degraded modes, watchdog/supervision behavior | Needed because replacement is behavioral, not only electrical | ☐ |
| Diagnostics | Fault codes, DTC mapping, tester-visible status, event logging, clear/reset rules | Needed for service tooling and verification | ☐ |

3. Interface Detail Tables (Fill-In)

A. Tester / AkaTool ↔ BMU message details

| Message / Command | Direction | Interface | ID / Service | Payload fields / scaling | Trigger / timing | Expected response / notes |
| --- | --- | --- | --- | --- | --- | --- |

B. BMU ↔ CSC (ADBMS6833 stack) details

| Function | Direction | Physical link | Sequence / state | Key parameters | Error handling | Reference doc / owner |
| --- | --- | --- | --- | --- | --- | --- |

C. EEPROM / NVM details

| Data block / area | Address / range | Meaning | Format / units | Written by | Read by | Validation / CRC / notes |
| --- | --- | --- | --- | --- | --- | --- |

4. Suggested Attachments to Request

Current BMU–tester message definition or DBC.

CSC schematic set and ADBMS6833 configuration details.

EEPROM / NVM layout, parameter list, and manufacturing defaults.

Startup, balancing, and fault-handling sequence descriptions.

Any existing software interface specification, test scripts, or service procedures.

Prepared as a reusable template for interface-definition discussions.